TASK-2

Execute the following the network commands like ipconfig, tracert, tellnet, netsh, ping, nslookup, netstat

Procedure

Step-1:

Launch cisco packet tracer

Double the cisco packet tracer icon on your desktop or find it in your applications list and open the program

Step-2:

Create a simple network topology

- Add devices

- Drag and drop a router and a switch from the device list on to the workspace

- Drag and drop two pc’s on to the workspace

- Connect devices
Use the connection tool to connect the devices

- Connect one pc to the switch using the copper straight-through

- Connect the switch to the router using another copper straight-through cable

- Connect the second pc to the switch using copper straight-through cable

- Configure devices

Step-3:

Click on the router

-Go to config tab
-Assign IP addresses to the router interfaces 
Eg: PC-0:
Interface G0/0: ip address 192.168.1.1, Subnet mask: 255.255.255.0
PC-1:
InterfaceG0/1: ip address 192.168.2.1, Subnet mask: 255.255.255.0

-Configure the pc
-Click on each pc 
-go to the desktop option and then ip configuration
-assign ip addresses to each pc 
Eg: PC0- 
IP address 192.168.1.2, Subset mask: 255.255.255.0, Default gateway: 192.168.1.1

Step-4:

- Execute networking commands

- click on a pc0

- go to the desktop tab

- open the command prompt for pc0
Command1: ipconfig
This command displays all correct tcp ip network configuration values and all dh cp and pns settings

Step5: Type tracert 
This command traces the path taken to a destination by sending icmp echo request messages

Step6: telnet
Go to router and click on CLI and write below steps

Go to pc -0 then desktop then command prompt then write telnet

Step7: IP BREIF INTERFACE

Type show ip interface brief to get below result

Step8: PING:

Type ping 192.168.2.2

Step9: NSLOOKUP:

Step1 : take a server and connect it and user copper-straight wire

Step2: go to server 0
             Then click on services
             Click on DNS and click on “ON”

Then go to pc0 then click on desktop then type as below:

Step10: Netstat

This command displays network connections for the Transmission Control Protocol (TCP), routing tables, and a number of network interface and network protocol statistics.

The netstat command is used to display network connections, routing tables, interface statistics, masquerade connections, and multicast memberships.